Revision change information

Assembly R1A → R1B
Board R1B → R1C

# General

This document describes the changes which have been done for <product> between product revisions R1A and R1B, not only on the circuit board but in production documentation overall. The circuit board is updated from R1B to R1C.

## REVISION HISTORY

| Revision | Comments | Date |
| --- | --- | --- |
| PA1 | First document revision |  |

## Consequences

Board revision R1C is very similar to R1B and for system users nothing has changed in the product specification. The circuit board layout has been updated and some components added/removed/moved so R1B PCBs can’t be used to produce R1C units. A general review of documentation has been done to remove obsolete elements.

## Reasons for changes

### Electrical performance

Documentation has been updated to standardize changes that were previously patched.

### Mechanical performance

Updates have been done in base plate milling (not in cast) to pass vibration tests.

# Changes of documentation

## Related documents

See document lists:

PNR_ Document list assy R1B

PNR_Document list board R1C

## Product revision information

The new revision of Assembly is R1B.

The new revision of the circuit board is R1C.

## Affected documents

Document	Document number	Old revision	New rev
Schematic	Pnr-411	PB1	B

BOM	Pnr.1BOMyymmdd	160203	160916

Inductor Spec 	EKD0220-204 	B	C

Test specification	Pnr-231	PA10	PA12

Assembly instruction	Pnr-225	A	B

Assembly instruction CB*	KPnr-225 CB*	B	C

Base plate dwg	Pnr-004	A	B

*CB = circuit board

# Board

## Changes in BOM

See Pnr_BOM compare 160203-160916.xlsx.

## Note on inductor IBC123

The nominal value of this inductor has been changed from 40µH to 44µH, however nothing has changed in the manufacturing specification. The previous nominal value was wrong and has been corrected.

## Layout changes

### Inductor moved

The inductor L1902 that was previously placed on the secondary side has been moved to primary side to avoid breaking.

### Capacitor footprint corrected

C1100 and C1101 have previously been patched to the board, now they have correct footprints.

### Diode rotated

The diode D1805 has been rotated 90 degrees because it used to be a little too close to the board edge.

### Addition of filtering

A few small components have been added to approve filtering (C1870, C2050, R1870).

### Heat relief

Some copper has been removed from the connection of R1100 to avoid soldering problems.

### Board thickness

The PCB has been made thicker to be more stable; it was 1.73mm before, now it’s 2.1mm.

### Board outline

The board outline has been altered to offer more vibration support.

## Test points

No test points have been moved.

# Mechanics Assembly

The base plate cast is unchanged but the milling has been altered:

Adaptations to new board outline.

The surface supporting alumina insulators has been lowered so that the alumina top surface will be in accordance with the soldering jig used for mounting semi-conductors.